Annex F:  Filling and Sealing the Pratt Pouch with 2.0 ml of Nevirapine (NVP)

Plastic bags

Fig. 2

Syringe

Plunger

Barrel

Needle hub

Lip

Syringe Tip

Fig. 1

Mothers’ Instruction Sheet

Pharmacy Identification Card

Part 2: Filling the Syringe

2.1 	Remove syringe from packaging as shown (Figure 1).

2.2  	Using syringe without a tip, draw NVP from bottle into syringe (Figure 2). 
It may be necessary to tip the bottle.

Note: Use only unopened medication bottles.

Pen

Preprinted / Handwritten labels

Nevirapine

Opening

Tearing notches

Pratt Pouch

Handle

Heating element

Timer knob

Indicator
light

Heat Sealer

Tissues

Wash and dry hands well before starting process.

Part 1: Assemble all Supplies

2.3  	Look closely at the NVP in syringe to check for air bubbles. If bubbles are present, carefully expel medication back into bottle and fill syringe again. 
Repeat until no air bubbles are seen in syringe.

Repeat until no air bubbles are seen in syringe.

Fig. 5

Fig. 3

2.6 	Plunge NVP from syringe back into bottle.

Note: steps 2.2—2.6 help ensure that the NVP remaining in needle tip does not contain air bubbles.

2.7 	Using syringe with tip in place, draw NVP from bottle into syringe. 
Close NVP bottle after use.
Note: The usual infant dose of NVP is 2.0 ml. Filling pouch to 2.2 ml helps ensure the infant receives a full dose even though residual NVP will remain in pouch. 
Note: Fill syringe carefully to ensure it contains the correct dose 2.0 ml). 
If too much NVP is placed in syringe, plunge the excess back into bottle.

2.4  	With a clean dry tissue, wipe excess NVP off needle hub.

2.5 	Push syringe tip onto needle hub of syringe (Figure 3). 
Ensure that the tip is secured tightly on the hub so that it will not loosen.

Part 3: Filling the Pratt Pouch

3.1 	Fill syringe with NVP according to instructions in Part 2 2.0ML in fig (Figure 4).

3.3 	Gently slide syringe all the way to bottom of pouch.

3.2 	Open pouch by sliding syringe tip into open end (Figure 5).

Note: It may help to place an index finger along back of pouch opening 
to stabilize it.
Note: Never touch inside of pouch with fingers, pencils, pens, screwdrivers 
or other objects as this can damage the pouch or contaminate the contents!

2.8 	With a clean dry tissue, wipe excess NVP off syringe tip.
Note: This is essential since any medication remaining on syringe tip can contaminate the top 
of the pouch during filling and then cause interference with the heat sealing process.

Fig. 4

3.5  	Slowly remove syringe from pouch. Avoid getting NVP on the sides of pouch.

Note: If small amount of NVP gets on outside of pouch, wipe it off completely and proceed with sealing.

3.4  	Dispense NVP into bottom of pouch by pushing plunger all the way down while 
slowly withdrawing syringe from pouch (this avoids getting medication on syringe tip).

Note: NVP should fill the bottom of pouch with little of the medication on the sides.

Fig. 6

Fig. 8

4.3  	Bend open end of filled pouch to help close the opening (Figure 6).

4.4  	Place pouch on heating element of sealer and hold in place as shown (Figure 7). Sealer should be touching pouch close to open end.

4.5  	Press handle of sealer all the way down onto pouch and hold in place.

Note: Be sure to press handle all the way down onto heating element. 
Failure to do this may result in an inadequate seal, making pouch unusable!

4.1  	Fill pouch with NVP according to instructions in Part 3.

Part 4: Sealing and Packaging the Pratt Pouch

4.2  	Plug in heat sealer; turn timer knob to maximum setting.

Note: The sealer heats only when handle is depressed. Power to the heating element automatically turns off when operator’s hand is removed from handle or when required 
time has elapsed.

Fig. 7

4.6  	Hold sealer handle down until red indicator light turns off and click is heard. This should take 1-3 seconds (Figure 8).

Note: Be sure that pouch remains on heating element for required amount of time! Early removal may result in inadequate seal, making pouch unusable!

4.7  	Remove pouch from sealer.

4.10 	Affix one label to each side of pouch, placing them below tearing notches so they do not interfere with opening pouch (Figure 10).

4.9 	Write necessary information on large label according to local requirements.

Note: Do not write on pouch itself (empty or filled) or on label after it is placed on pouch as this can damage the foil and contaminate the contents. 
Note: The small label is preprinted; with drug name, dosage and expiration date; no additional written information is not necessary.

Note: The medication in the pouch is stable for one (1) year following sealing.  The expiration date on the pouches is determined by the expiration date of the original medication bottle.

Fig. 11

Fig. 9

4.8  	Test seal by sliding thumb and forefinger firmly up the pouch toward sealed end. If any medication oozes out from pouch, the seal was not sufficient and pouch should be discarded (Figure 9).

Note: Test the seal on every pouch in this way.

Fig. 10

To fill additional pouches, repeat process starting with step 2.7 above.

At the end of each pouch filling/sealing session, dispose of syringe and syringe tip. Use new supplies for subsequent sessions.

Wash hands well when finished.

Store filled pouches under conditions not exceeding 40°C.

4.11 	Place filled, sealed and labeled pouches in the pharmacy bags provided for this use (Figure 11). Also include the pharmacy identification card and the mothers’ instruction sheet (Annex M).